USER MANUAL

1. INTRODUCTION

Thank you for choosing the Acue Lighting Halo Beam 150. In order to use this product correctly and safely, please read the instructions carefully before installing and using this product. This manual contains important installation and operation information. Make sure to strictly follow the instructions when installing and operating this product. Retain this manual in a safe place for future reference.

The Halo Beam 150 uses a new high-temperature resistant metal and plastic body. This product is designed and produced in strict accordance with CE standards, and conforms to the international standard of DMX512 signal protocol. It can be controlled manually from the display or through DMX using the built in 3 pin XLR in/out connectors. This fixture has multiple DMX channel modes to choose from. It is suitable for various types of concerts, theaters, studios, nightclubs, bars and other like environments.

2. Safety Instructions

- Please keep this User Manual for future reference. All users of this fixture must read the user manual thoroughly.

- Unpack and check contents carefully to make sure there is no damage caused by shipping before using the unit.

- Before operating, ensure that the voltage and frequency of power supply match the power requirements of the unit.

- It’s important to properly ground the fixture to avoid electric shock.

- The unit is for indoor use only. Use only in a dry location

- The unit must be installed in a location with adequate ventilation, at least 50cm from adjacent surfaces. Be sure that no ventilation slots are blocked

- Disconnect main power before moving fixture.

- Make sure there are no flammable materials close to the unit while operating, as it is fire hazard.

- Use safety cable when mounting this unit. Don’t handle the unit by taking its arms or head. The fixture may only be moved using the built in handles or by holding from the base.

- Maximum ambient operating temperature is :40˚C. Do not operate if the temperature is higher than this.

- Unit surface temperature may reach up to 85˚C. Do not touch the housing bare-hand during its operation. Turn off the power and allow about 15 minutes for the unit to cool down before replacing or serving.

- In the event of serious operating problem, stop using the unit immediately. Never try to repair the unit by yourself. Repairs carried out by unskilled individuals can lead to further damage or malfunction. Please contact the nearest authorized technical assistance center. Always use the same type of spare parts.

## 3. Maintenance and Installation

## 3.1  Maintenance

- To reduce the risk of electrical shock or fire, do not expose this unit to rain or moisture.

- Intermittent usage will extend this fixtures service life.

- Please clear the fan, fan net, and optical lens of any dust or obstructions in order to keep the fixture in a proper work state.

- Do not use alcohol or any other organic solvent to wipe the shell.

## 3.2  Statement

All fixtures have been tested thoroughly before leaving the factory. In order to keep the product in good condition and ensure safe operation, users should follow the safety precautions and warnings in this manual.

Important: Damages caused by not following the guidance in this user manual are not covered under warranty. The manufacturer is not responsible for product issues caused by misuse.

## 3.3  Safety Precaution

- In order to guarantee the product’s life expectancy, do not store or use in damp places or environments over 60˚C.

- Always mount this unit in a safe and stable manner. Mounting system must be able to withstand 10x that of the fixture’s weight. A safety cable must always be used as a secondary source for securing the fixture.

- Skilled individuals must only perform the installation of this fixture.

- This product can be used in the voltage range of 90-240V and is an indoor product. Please make sure that the power mains voltage used is not higher/or lower than what the product can withstand! ! The power plug must be inserted into a protected Class I socket and must be grounded properly.

- In order to make sure the product is used properly, please read this manual carefully.

## 3.4  Cable connection (DMX)

Make sure to use a cable conforming to specifications EIA RS-485: 2-pole twisted, shielded, 120Ohm characteristic impedance, 22-24 AWG, Low capacity. Do not use microphone cable or other cable with characteristics differing from those specified. The end connections must be made using XLR type 3 or 5 pin male/female connectors. A terminating plug   must be inserted into the last fixture with a resistance of 120 Ohm (minimum 1/4 W) between terminals 2 and  3.

IMPORTANT: The wires must not make contact with each other or with the metal casing of the connectors. The casing itself must be connected to the ground of the connector plugs.

## 3.5  Rigging (Optional)

This equipment can be positioned and fixed by a clamp in every direction of the stage. The included locking system makes it easy to fasten to the bracket.

Attention! Two clamps are required to mount the equipment. Every clamp is locked to the fixture by the included omega fastener. The Omega fastener can only be locked clockwise.

Attention! A safety cable is also required when hanging the fixture. The safety cables is a secondary source of safety when hanging fixtures.

***BEFORE HANGING FIXTURE***

- Check all rigging clamps for damages. Discard clamp if damaged and replace with a properly functioning clamp. Make sure the hanging point/structure can support ten times the weight of all the equipment, clamps, wiring and other fixtures mounted to it.

- Make sure to tie the screws of the clamp firmly. Use one M12 screw (Grade 8.8 or higher) to clamp bracket, and then screw on the nuts.

- Install a safety cable, which can withstand at least ten times the weight of the fixture.

- You must position the fixture at least 3 feet away from flammable materials or lighting source. Low capacity

- The casing itself must be connected to the ground of the connector plugs.

# 4. Display Operation Menu

| Item | Menu | Description |
| --- | --- | --- |
| Display | ADD 001 DMX | DMX Address |
| 1 | Address | Address setup (1-494) |
| 2 | Run Mode | Operation Model (DMX/Auto Run/Sound) |
| 3 | Channel | Channel model (16CH) |
| 4 | Invert-X | X axis reverse moving |
| 5 | Invert-Y | Y axis reverse moving |
| 6 | Pan | X axis starting position calibration |
| 7 | Title | Y axis starting position calibration |
| 8 | Focus | Focus starting position calibration |
| 9 | Color | Color starting position calibration |
| 10 | Gobo | Gobo starting position calibration |
| 11 | Prism | Prism starting position calibration |
| 12 | Dimmer | Dimmer from 0- 100% |
| 13 | Bklight | Setup the backlight time |
| 14 | Sound | Sound sensitivity setup |
| 15 | Default | Restore the default value |
| 16 | Reset | Reset Fixture |
| 17 | Version | Version details |

# 5. DMX Channel Chart

| Item | Function | Range | Description |
| --- | --- | --- | --- |
| 1 | X | 0-255 | 0--540° |
| 2 | X Fine-tuning | 0-255 | X axis 16bit |
| 3 | Y | 0-255 | 0--270° |
| 4 | Y Fine-tuning | 0-255 | Y axis 16bit |
| 5 | X/Y speed adjustment | 0-255 | Fast --- Slow |
| 6 | Dimmer | 0-255 | 0%-100% Dimmer |
| 7 | Strobe | 0-3 | Shutter Closed |
|  |  | 4-99 | Synchronous strobe |
|  |  | 100-149 | Pulse strobe |
|  |  | 150-199 | Strobe |
|  |  | 200-250 | Random strobe |
|  |  | 251-255 | Shutter Open |
| 8 | Color | 0-4 | White |
|  |  | 5-9 | Color 1 |
|  |  | 10-14 | Color 2 |
|  |  | 15-19 | Color 3 |
|  |  | 20-24 | Color 4 |
|  |  | 25-29 | Color 5 |
|  |  | 30-34 | Color 6 |
|  |  | 35-39 | Color 7 |
|  |  | 40-44 | Color 8 |
|  |  | 45-49 | Color 9 |
|  |  | 50-54 | Color 10 |
|  |  | 55-59 | Color 11 |
|  |  | 60-64 | Color 12 |
|  |  | 65-69 | Color 13 |
|  |  | 70-74 | White + Color 1 |
|  |  | 75-79 | Color 1 + Color 2 |
|  |  | 80-84 | Color 2 + Color 3 |
|  |  | 85-89 | Color 3 + Color 4 |
|  |  | 90-94 | Color 4 + Color 5 |
|  |  | 95-99 | Color 5 + Color 6 |
|  |  | 100-104 | Color 6 + Color 7 |
|  |  | 105-109 | Color 7 + Color 8 |
|  |  | 110-114 | Color 8 + Color 9 |
|  |  | 115-119 | Color 9 + Color 10 |
|  |  | 120-124 | Color 10 + Color 11 |

|  |  | 125-129 | Color 11 + Color 12 |
| --- | --- | --- | --- |
|  |  | 130-134 | Color 12 + Color 13 |
|  |  | 135-139 | Color 13 + White |
|  |  | 140-195 | Clockwise rotation from fast to slow |
|  |  | 196-199 | Stop Rotation |
|  |  | 200-255 | Anti- Clockwise rotation from slow to fast |
| 9 | Gobo | 0-9 | White |
|  |  | 10-13 | Gobo 1 |
|  |  | 14-17 | Gobo 2 |
|  |  | 18-21 | Gobo 3 |
|  |  | 22-25 | Gobo 4 |
|  |  | 26-29 | Gobo 5 |
|  |  | 30-33 | Gobo 6 |
|  |  | 34-37 | Gobo 7 |
|  |  | 38-41 | Gobo 8 |
|  |  | 42-45 | Gobo 9 |
|  |  | 46-49 | Gobo 10 |
|  |  | 50-53 | Gobo 11 |
|  |  | 54-57 | Gobo 12 |
|  |  | 58-61 | Gobo 13 |
|  |  | 62-65 | Gobo 14 |
|  |  | 66-69 | Gobo 15 |
|  |  | 70-77 | Gobo 1 Shake |
|  |  | 78-85 | Gobo 2 Shake |
|  |  | 86-93 | Gobo 3 Shake |
|  |  | 94-101 | Gobo 4 Shake |
|  |  | 102-109 | Gobo 5 Shake |
|  |  | 110-117 | Gobo 6 Shake |
|  |  | 118-125 | Gobo 7 Shake |
|  |  | 126-133 | Gobo 8 Shake |
|  |  | 134-141 | Gobo 9 Shake |
|  |  | 142-149 | Gobo 10 Shake |
|  |  | 150-157 | Gobo 11 Shake |
|  |  | 158-165 | Gobo 12 Shake |
|  |  | 166-173 | Gobo 13 Shake |
|  |  | 174-181 | Gobo 14 Shake |
|  |  | 182-189 | Gobo 15 Shake |
|  |  | 190-221 | Counter clockwise rotation from fast to slow |
|  |  | 222-223 | Stop rotation |
|  |  | 224-255 | Clockwise rotation from slow to fast |

31

| 10 | Prism | 0 |  | Prism off |
| --- | --- | --- | --- | --- |
|  |  | 32-63 |  | Prism on |
|  |  | 64-255 |  | Slow to fast rotation Counter-clockwise |
| 11 | Focus | 0-255 |  | Linear focus |
| 12 | Dimmer | 0-255 |  | 0%-100% Dimmer |
| 13 | Strobe | 0-3 |  | Off |
|  |  | 4-255 |  | Strobe |
| 14 | SMD LED Ring Effect | 0-51 |  | Effect Off |
|  |  | 52-55 |  | Effect 1 |
|  |  | 56-59 |  | Effect 2 |
|  |  | … |  | … |
|  |  | 252-255 |  | Effect 51 |
| 15 | SMD LED Ring Effect Speed | 0-255 |  | From slow to fast |
| 16 | Reset | 0-199 |  | No function |
|  |  | 200-205 |  | Reset |
|  |  | 206-255 |  | No function |

## 6. Technical Specifications:

Input Voltage: AC110V-240V, 50/60Hz

Max Power Draw: 150W

Main Light Source: 1 X 100W White LED

LED RING Source: 30pcs X 0.2W RGB 3IN1

Color Wheel: 15 colors + open

Gobo Wheel: 13 gobos + open

Prism: 8 Facet Prism

Lens Angle: 2.5°

Channels: 16 CH

Operation Modes: DMX512, Sound, Auto, Master-slave

Dimmer: 0~100% linear dimmer

Pan scanning: 540°

Tilt scanning: 270°

For more information about this product as well as other products please visit www.AcueExpress.com

Or contact us directly:

Email: Info@AcueLighting.com

Phone: (818) 646-7526 / (818) 672-6822