USER MANUAL

1: INTRODUCTION

Thank you for choosing the Acue Lighting Illusion Bar. In order to use this product correctly and safely, please read the instructions carefully before installing and using this product. This manual contains important installation and application information. Make sure to strictly follow the instructions when installing and operating this product. Retain this manual in a safe place for future reference.

The Illusion Bar uses a new high-temperature resistant metal body. This product is designed and produced in strict accordance with CE standards, and conforms to the international standard of DMX512 signal protocol. It can be controlled manually from the display or through DMX using the built in 3 pin XLR in/out connectors. This fixture has multiple DMX channel modes to choose from. It is suitable for various types of concerts, theaters, studios, nightclubs, bars and other like environments.

2. Getting Started

Carefully remove the fixture from its packaging. Check whether the product has been damaged during transportation, and check whether the following contents are complete.

1x Illusion Bar Fixture

1x Power cord

1x User manual

All fixtures have been tested thoroughly before leaving the factory. In order to keep the product in good condition and ensure safe operation, users should follow the safety precautions and warnings in this manual.

Important: Damages caused by not following the guidance in this user manual are not covered under warranty. The manufacturer is not responsible for product issues caused by misuse.

If the product has been exposed to extreme temperature environments during transportation or storage, please do not connect the product to the power mains immediately. High Heat as well as condensation or moisture due to temperature changes may damage the product. Only use it after the product has returned to normal temperature and after the product has been inspected for moisture. Maximum ambient operating temperature is 40˚C. Do not operate it where the temperature is higher than this.

This product can be used in the voltage range of 90-240V and is an indoor product. Please make sure that the power mains voltage used is not higher than what the product can withstand! ! The power plug must be inserted into a protected Class I socket and must be grounded properly.

3: Connection of DMX512

The fixture uses the DMX512 signal control protocol with the ability to link multiple fixtures together using the built in 3-pin XLR connections located in the back of the fixture. When connecting the signal to multiple fixtures, it is best to use a double-core shielded DMX cable. Each fixture is connected through the internal DMX signal jacks (XLR Sockets) INPUT (input) and OUTPUT (output), and the 3-pin XLR plugs of fixture must correspond the order of fixtures. It is recommended to use a DMX signal terminator for the last fixture in the chain.

4: Display Functions

- Button Description

2 Digital tube display menu description

| serial number | content | Value range | Function and description |
| --- | --- | --- | --- |
| 1 | A001 | 001- 512 | DMX512 address code setting |
| 2 | 8CH | 2/4/6/8/112/11 4 | Channel mode settings. |
| 3 | R255 | 000 - 255 | Red dimming, from dark to bright. |
| 4 | G255 | 000 - 255 | Green dimming, from dark to bright. |
| 5 | b255 | 000 - 255 | Blue dimming, from dark to bright |
| 6 | W255 | 000 - 255 | White dimming, from dark to bright. |
| 7 | F000 | 000 - 255 | RGBW strobe, speed from slow to fast. |
| 8 | CL16 | 01 – 16 | 8 color options + 8 breathing effects |
| 9 | CC09 | 01 – 15 | color jump effect |
| 10 | dE09 | 01 – 15 | color gradient effect |
| 11 | CP09 | 01 – 15 | color pulse effect |
| 12 | EF01 | 01 - 15 | 15 built-in effects to choose from |
| 12 | SP09 | 01 - 09 | Built- in effect speed control |
| 14 | Sou1 | 00 - 01 | 00 is full color voice control, 01 is effect voice control |

# 5. DMX Channel Tables

2 channel mode

| channel | DMX value | Function Description |
| --- | --- | --- |
| CH1 | 000-009 | no effect |
|  | 010-021 | LED color selection 2 channel selection color |
|  | 022-027 | red breathing effect |
|  | 028-033 | green breathing effect |
|  | 034-039 | blue breathing effect |
|  | 040-045 | white breathing effect |
|  | 046-051 | yellow breathing effect |
|  | 052-057 | purple breathing effect |
|  | 058-061 | Cyan breathing effect |
|  | 064-069 | Circulatory breathing effect |
|  | 070-079 | jump effect |
|  | 080-089 | Gradient effect |
|  | 090-099 | pulsation effect |
|  | 100- 109 | Horse racing effect - 1 |
|  | 110- 119 | Horse racing effect-2 |
|  | 120- 129 | Horse racing effect-3 |
|  | 130- 139 | Horse racing effect-4 |
|  | 140- 149 | Horse racing effect- 5 |
|  | 150- 159 | Horse racing effect- 6 |
|  | 160- 169 | Horse racing effect- 7 |
|  | 170- 179 | Horse racing effect- 8 |
|  | 180- 189 | Horse racing effect- 9 |
|  | 190- 199 | Horse racing effect- 10 |
|  | 200-209 | Horse racing effect- 11 |
|  | 210-219 | Horse racing effect- 12 |
|  | 220-229 | Horse racing effect- 13 |
|  | 230-239 | Horse racing effect- 14 |
|  | 240-249 | Horse racing combination effect- 1- 14 |

|  | 250-255 | voice control effect |
| --- | --- | --- |
| CH2 | 000-255 | Function speed, from slow to fast In voice control mode, 0- 127 is full color voice control, 127-255 is effect voice control |

4  channel mode

| channel | Function | Description |
| --- | --- | --- |
| CH1 | 000-255 | R red dimming, from dark to bright |
| CH2 | 000-255 | G green dimming, from dark to bright |
| CH3 | 000-255 | B blue dimming, from dark to bright |
| CH4 | 000-255 | W white dimming, from dark to bright |

6  channel mode

| channel | Function | Description |
| --- | --- | --- |
| CH1 | 000-255 | RGBW total dimming, from dark to bright |
| CH2 | 005-255 | RGBW total strobe |
| CH3 | 000-255 | R red dimming, from dark to bright |
| CH4 | 000-255 | G green dimming, from dark to bright |
| CH5 | 000-255 | B blue dimming, from dark to bright |
| CH6 | 000-255 | W white dimming, from dark to bright |

8  channel mode

| channel | Function | Description |
| --- | --- | --- |
| CH1 | 000-255 | RGBW total dimming, from dark to bright |
| CH2 | 005-255 | RGBW strobe |
| CH3 | 000-009 | no effect |
|  | 010-021 | LED color selection 2 channel selection color |
|  | 022-027 | red breathing effect |
|  | 028-033 | green breathing effect |
|  | 034-039 | blue breathing effect |
|  | 040-045 | white breathing effect |
|  | 046-051 | yellow breathing effect |
|  | 052-057 | purple breathing effect |
|  | 058-061 | Cyan breathing effect |
|  | 064-069 | Circulatory breathing effect |
|  | 070-079 | jump effect |
|  | 080-089 | Gradient effect |
|  | 090-099 | pulsation effect |
|  | 100- 109 | Horse racing effect - 1 |
|  | 110- 119 | Horse racing effect-2 |
|  | 120- 129 | Horse racing effect-3 |
|  | 130- 139 | Horse racing effect-4 |
|  | 140- 149 | Horse racing effect- 5 |
|  | 150- 159 | Horse racing effect- 6 |
|  | 160- 169 | Horse racing effect-7 |

|  | 170- 179 | Horse racing effect- 8 |
| --- | --- | --- |
|  | 180- 189 | Horse racing effect- 9 |
|  | 190- 199 | Horse racing effect- 10 |
|  | 200-209 | Horse racing effect- 11 |
|  | 210-219 | Horse racing effect- 12 |
|  | 220-229 | Horse racing effect- 13 |
|  | 230-239 | Horse racing effect- 14 |
|  | 240-249 | Horse racing combination effect- 1- 14 |
|  | 250-255 | voice control effect |
| CH4 | 000-255 | Function speed, from slow to fast |
| CH5 | 000-255 | In voice control mode, 0- 127 is full color voice control, 127-255 is effect voice control |
| CH6 | 000-255 | G green dimming, from dark to bright |
| CH7 | 000-255 | B blue dimming, from dark to bright |
| CH8 | 000-255 | W white dimming, from dark to bright |

112 channel mode

| channel | Function | Description |
| --- | --- | --- |
| CH1 | 000-255 | R1 red dimming, from dark to bright |
| CH2 | 000-255 | G1 green dimming, from dark to bright |
| CH3 | 000-255 | B1 blue dimming, from dark to bright |
| CH4 | 000-255 | W1 white dimming, from dark to bright |
| CH5 | 000-255 | R2 red dimming, from dark to bright |
| CH6 | 000-255 | G2 green dimming, from dark to bright |
| CH7 | 000-255 | B2 blue dimming, from dark to bright |
| CH8 | 000-255 | W2 white dimming, from dark to bright |
| … | … | … |
| … | … | … |
| … | … | … |
| … | … | … |
| CH105 | 000-255 | R27 red dimming, from dark to bright |
| CH106 | 000-255 | G27 green dimming, from dark to bright |
| CH107 | 000-255 | B27 blue dimming, from dark to bright |
| CH108 | 000-255 | W27 white dimming, from dark to bright |
| CH109 | 000-255 | R28 red dimming, from dark to bright |
| CH110 | 000-255 | G28 green dimming, from dark to bright |
| CH111 | 000-255 | B28 blue dimming, from dark to bright |
| CH112 | 000-255 | W28 white dimming, from dark to bright |

114 channel mode

| channel | Function | Description |
| --- | --- | --- |
| CH1 | 000-255 | RGBW total dimming, from dark to bright |
| CH2 | 000-255 | RGBW strobe |
| CH3 | 000-255 | R1 red dimming, from dark to bright |
| CH4 | 000-255 | G1 green dimming, from dark to bright |
| CH5 | 000-255 | B1 blue dimming, from dark to bright |
| CH6 | 000-255 | W1 white dimming, from dark to bright |

| CH7 | 000-255 | R2 red dimming, from dark to bright |
| --- | --- | --- |
| CH8 | 000-255 | G2 green dimming, from dark to bright |
| CH9 | 000-255 | B2 blue light, from dark to bright |
| CH10 | 000-255 | W2 white dimming, from dark to bright |
| … | … | … |
| … | … | … |
| … | … | … |
| … | … | … |
| CH107 | 000-255 | R27 red dimming, from dark to bright |
| CH108 | 000-255 | G27 green dimming, from dark to bright |
| CH109 | 000-255 | B27 blue dimming, from dark to bright |
| CH110 | 000-255 | W27 white dimming, from dark to bright |
| CH111 | 000-255 | R28 red dimming, from dark to bright |
| CH112 | 000-255 | G28 green dimming, from dark to bright |
| CH113 | 000-255 | B28 blue dimming, from dark to bright |
| CH114 | 000-255 | W28 white dimming, from dark to bright |

# 6. Technical Specifications

Input Voltage: AC110-240V 50/60HZ

Light Source: 168PCS*0.5W RGB 3in1 SMD 5050 LED + 168PCS*0.5W White SMD 5050 LED

Effect: Wash , Running , Strobe 3 effect

Control Mode: DMX512 ，master-salve, sound, auto

Power：150 W 3pcs led as a group for Point control

For more information about this product as well as other products please visit www.AcueExpress.com

Or contact us directly:

Email: Info@AcueLighting.com

Phone: (818) 646-7526/ (818) 672-6822